## Customization Summary: Mobile Robot URDF Design

### 1. Structural Components (Links)

The robot was built with five links forming a tricycle-style differential drive structure with an omnidirectional front caster:

- Base Link: A blue rectangular prism (0.4 × 0.3 × 0.1) acting as the main chassis.

- Left Rear Wheel: A black cylinder (Radius: 0.1, Length: 0.05) positioned on the left side of the chassis for differential drive locomotion.

- Right Rear Wheel: A black cylinder (Radius: 0.1, Length: 0.05) positioned on the right side of the chassis for differential drive locomotion.

- Front Omnidirectional Wheel: A grey sphere (Radius: 0.05) mounted under the front-center of the chassis, acting as a passive caster wheel for balance and smooth turning.

- LiDAR Sensor: A red cylinder (Radius: 0.05, Length: 0.04) mounted at the front-top of the chassis for obstacle detection and environment scanning.

### 2. Joint Configurations

Four joints define the relationship between all links:

- Continuous Joints: Applied to left_wheel_joint, right_wheel_joint, and front_omni_joint. These allow infinite rotation around their respective axes — essential for forward, backward, and turning motion in simulation.

- Fixed Joint: Applied to lidar_joint to rigidly attach the LiDAR sensor to the base_link. As a sensor, its position relative to the chassis remains constant at all times.

### 3. Coordinate Frames & Transformations (TF)

The robot's transformation tree was configured with the following precise offsets:

- Root Frame: base_link is set as the fixed frame for the entire system.

- Left Wheel Transform: Offset at [-0.1, 0.175, 0.0] with a 1.57 rad rotation on the X-axis, placing it flush on the left rear side.

- Right Wheel Transform: Offset at [-0.1, -0.175, 0.0] with a 1.57 rad rotation on the X-axis, placing it flush on the right rear side.

- Front Omni Wheel Transform: Offset at [0.15, 0.0, -0.05] — the Z offset of -0.05 compensates for the smaller radius (0.05) so the wheel bottom touches the same ground plane as the rear wheels (z = -0.1).

- LiDAR Sensor Transform: Offset at [0.15, 0.0, 0.075] with no rotation, placing it flush on the front-top edge of the chassis.

- Wheel Orientation: All drive wheel joints are rotated 1.57 radians (90°) on the X-axis so the cylinders are oriented correctly for rolling along the ground.

### 4. Ground Contact Consistency

All three wheels are precisely aligned to touch the ground at the same Z level (z = -0.1):

| Wheel | Radius | Joint Z | Ground Contact |
| --- | --- | --- | --- |
| Left Rear | 0.1 | 0.0 | 0.0 − 0.1 = −0.1 ✅ |
| Right Rear | 0.1 | 0.0 | 0.0 − 0.1 = −0.1 ✅ |
| Front Omni | 0.05 | −0.05 | −0.05 − 0.05 = −0.1 ✅ |

### 5. Visualization Performance

- RViz Integration: The model correctly renders in RViz with all colors (Blue chassis, Black rear wheels, Grey front caster, Red LiDAR) and geometries visible.

- TF Tree Integrity: All four child links (left_wheel, right_wheel, front_omni_wheel, lidar_sensor) are correctly parented to base_link, forming a clean single-root TF tree.

- Geometry Validity: The invalid length attribute was removed from the front omni wheel's <sphere> geometry, ensuring clean URDF parsing with no warnings.