MCT-454L  Mobile Robotics  |  ros2_ws_bilal

Deliverable 1

Lab Report

Lab Manual 5: Introduction to Gazebo and RViz with ROS 2 using TurtleBot3

# 1. Introduction

This report documents the step-by-step process followed during Lab Manual 5 of MCT-454L Mobile Robotics. The lab introduced simulation and visualization of the TurtleBot3 robot using Gazebo and RViz under ROS 2 Humble. The workspace used throughout this lab was ros2_ws_bilal.

# 2. Environment Setup

## 2.1  Package Installation

The system packages were updated and the required TurtleBot3 and Gazebo ROS 2 packages were installed using the following commands:

sudo apt update

sudo apt install ros-humble-turtlebot3* ros-humble-gazebo-ros-pkgs

## 2.2  Workspace and Environment Configuration

The ROS 2 environment and the ros2_ws_bilal workspace were sourced, and the TurtleBot3 model was permanently set to burger by adding the following lines to ~/.bashrc:

source /opt/ros/humble/setup.bash

source ~/ros2_ws_bilal/install/setup.bash

export TURTLEBOT3_MODEL=burger

The bashrc was reloaded with source ~/.bashrc to apply changes immediately.

# 3. Steps Followed to Complete Each Task

## Task 1 — Navigate Robot to Target Point

Gazebo was launched in Terminal 2 using:

ros2 launch turtlebot3_gazebo turtlebot3_world.launch.py

The keyboard teleoperation node was launched in Terminal 5:

ros2 run turtlebot3_teleop teleop_keyboard

The robot was navigated to the designated target point using w (forward), a (left), d (right), and s (stop) keys.

## Task 2 — Observe LiDAR Scan and Odometry in RViz

Cartographer and RViz were launched in Terminal 4:

ros2 launch turtlebot3_cartographer cartographer.launch.py use_sim_time:=true

The LaserScan and Odometry plugins were added in RViz via the Add button. The Fixed Frame was set to map. Real-time LiDAR scans were observed as colored dots surrounding the robot model, and odometry data was visualized as arrows showing robot pose and velocity.

## Task 3 — Explore TF Plugin

The TF (Transform) plugin was added in RViz. It visualized the coordinate frames of the robot including base_link, base_scan, odom, and map. The frames were observed as colored axes (X=red, Y=green, Z=blue). The base_link frame is the robot's body frame, child of odom, which is in turn child of the map frame, forming a parent-child TF tree.

## Task 4 — Customize Odometry Display

The Odometry plugin was added and customized in RViz. The topic was set to /odom. Arrow scale and color were adjusted for better visibility. Real-time robot motion was observed through the continuously updating arrow direction and magnitude, reflecting changes in linear and angular velocity as the robot was driven via teleoperation.

## Task 5 — Record and Replay with ros2 bag

A bags directory was created inside ros2_ws_bilal and all topics were recorded while the robot was driven:

cd ~/ros2_ws_bilal

mkdir -p bags

ros2 bag record -a -o bags/lab5_motion

Recording was stopped with Ctrl+C after navigating. The bag was replayed using ros2 bag play bags/lab5_motion and the data was visualized in RViz confirming all topics were recorded correctly.

## Task 6 — Save SLAM Map

After building a sufficient map with Cartographer, the map was saved using:

cd ~/ros2_ws_bilal

mkdir -p maps

ros2 run nav2_map_server map_saver_cli -f maps/my_map

This generated my_map.pgm (occupancy grid image) and my_map.yaml (metadata) in ~/ros2_ws_bilal/maps/.

## Task 7 — Return Robot to (0, 0, 0)

Using the teleoperation node, the robot was carefully driven back towards the origin coordinates (0, 0, 0). The Odometry display and map in RViz were monitored to guide the robot back to its starting position. The robot's position was verified using the /odom topic data.

## Task 8 — cmd_vel Publisher Node

A Python ROS 2 node was created at ~/ros2_ws_bilal/src/lab5_tasks/lab5_tasks/cmd_vel_publisher.py. The node uses a timer callback at 2-second intervals to alternate publishing forward velocity (linear.x = 0.2 m/s) and zero velocity (linear.x = 0.0) to the /cmd_vel topic. The node was run with:

python3 src/lab5_tasks/lab5_tasks/cmd_vel_publisher.py

The robot was observed alternating between moving forward and stopping every 2 seconds, confirming the publisher was working correctly.

## Task 9 — /odom Subscriber Node

The message type of the /odom topic was identified as nav_msgs/msg/Odometry by running ros2 topic info /odom. A Python subscriber node was created at ~/ros2_ws_bilal/src/lab5_tasks/lab5_tasks/odom_subscriber.py. The node subscribes to /odom and prints the robot position (x, y, z), orientation (z, w), and linear velocity in each callback. The node was run with:

python3 src/lab5_tasks/lab5_tasks/odom_subscriber.py

Continuous odometry messages were printed to the terminal, confirming successful subscription and message parsing.

MCT-454L Mobile Robotics  |  Lab Manual 5  |  ros2_ws_bilal